Water Resources Research

Supporting Information for

Numerical Simulation of Flow Resistance due to the Limnoperna fortunei Attachment

Qingqing Hou1, Chengfan Sun1, Yuncheng Xu1,2,3, and Mengzhen Xu4

1 College of Water Resources and Civil Engineering, China Agricultural University, Beijing, China, 2 Beijing Engineering Research Center of Safety and Energy Saving Technology for Water Supply Network System, Beijing, China, 3 Engineering Resources Center of Agricultural Water-Saving and Water Resources, Ministry of Education, Beijing, China. 4 State Key Laboratory of Hydroscience and Engineering, Tsinghua University, Beijing, China.

Corresponding author: Y. Xu (ycxu@cau.edu.cn)

M. Xu (mzxu@mail.tsinghua.edu.cn)

Contents of this file

Text S1. The change rate of Manning’s n value calculation

Figures S1. The geometry models of Limnoperna fortunei (LF) in the south and north of China at different densities

Table S1. The geometric parameters and hydraulic parameters of all the numerical cases performed in study

Introduction

This supporting information provides the change rate of manning roughness coefficient calculation, the geometry models of LF in the south and north of China at different densities, and the geometric parameters and hydraulic parameters of all the numerical cases performed in study.

Text S1. The change rate of Manning’s n value calculation

The change rate of Manning’s n value is calculated based on the numerical results at different LF attachment densities according to the following equation:

Where  is the change rate of the manning roughness coefficient (%);  is the manning roughness coefficient with the attachment of LF;  is the manning roughness coefficient without the attachment of LF.

Figure S1. The geometry models of LF in the south and north of China at different densities

All the geometry models were built based on real size of LF in Blender. Carefully considering the mean size of an individual adult LF in the south and north of China, a 20 cm × 20 cm rigid plane was selected to place a certain density of LF. The placement of LF on the plane were processed by activating the falling action of the physical motion engine in Blender. After completing the set of single model free-falling to the plane, attachment models were built by adding geometry nodes based on single model. During the setup of geometry nodes, the size of LF was adjusted according to scale vector on Instance on Points, the density was set on Distribute Points on Faces. Due to the maximum LF shell length limitation in the south, the highest achievable density for a single-layer LF is 15000 ind/m2. Any density exceeding this limit would result in multi-layer attachment. Similarly, the maximum density in the north is 20000 ind/m2. In addition, the location and orientations of each individual LF in the attachment model placed on the plane were randomized. The placement process for each density was replicated three times to yield three unique 20 cm × 20 cm LF attachment planes, with the intention of capturing a certain degree of bio-attachment diversity.

Figure S1 (a). The geometry model of LF in the south of China at different densities

Figure S1 (b). The geometry model of LF in the north of China at different densities (3 replicates)

Table S1. The geometric parameters and hydraulic parameters of all the numerical cases performed in study

| South (mean size of ind. model: 22.0 cm×9.2 cm×9.7 cm) |  |  |  |  |  |  |  |  |  | North (mean size of ind. model: 17.0 cm×7.3 cm×7.7 cm) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Density (ind/m2) | H0 (mm) | Hm (mm) | z (mm) | Cb (%) | Lc | H (m) | U0 (m/s) | n | n (%) | H0 (mm) | Hm (mm) | z (mm) | Cb (%) | Lc | H (m) | U0 (m/s) | n | n (%) |
| 0 | 0.0 | 0.0 | 0.0 | 0 | 0.000 | 0.5 | 0.20 | 0.0086 | 0 | 0.0 | 0.0 | 0.0 | 0 | 0.000 | 0.5 | 0.20 | 0.0086 | 0 |
|  |  |  |  |  |  |  | 0.25 | 0.0086 | 0 |  |  |  |  |  |  | 0.25 | 0.0084 | 0 |
|  |  |  |  |  |  |  | 0.30 | 0.0084 | 0 |  |  |  |  |  |  | 0.30 | 0.0082 | 0 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0101 | 0 |  |  |  |  |  | 5.0 | 0.20 | 0.0101 | 0 |
|  |  |  |  |  |  |  | 1.00 | 0.0086 | 0 |  |  |  |  |  |  | 1.00 | 0.0086 | 0 |
|  |  |  |  |  |  |  | 2.00 | 0.0082 | 0 |  |  |  |  |  |  | 2.00 | 0.0082 | 0 |
| 500 | 13.2 | 27.8 | 4.1 | 4 | 0.101 | 0.5 | 0.20 | 0.0142 | 66 | 9.2 | 20.3 | 2.9 | 1 | 0.068 | 0.5 | 0.20 | 0.0119 | 38 |
|  |  |  |  |  |  |  | 0.25 | 0.0145 | 73 |  |  |  |  |  |  | 0.25 | 0.0126 | 50 |
|  |  |  |  |  |  |  | 0.30 | 0.0148 | 80 |  |  |  |  |  |  | 0.30 | 0.0139 | 69 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0152 | 51 |  |  |  |  |  | 5.0 | 0.20 | 0.0141 | 40 |
|  |  |  |  |  |  |  | 1.00 | 0.0149 | 72 |  |  |  |  |  |  | 1.00 | 0.0092 | 7 |
|  |  |  |  |  |  |  | 2.00 | 0.0146 | 78 |  |  |  |  |  |  | 2.00 | 0.0077 | 6 |
| 1000 | 13.0 | 28.9 | 4.3 | 7 | 0.191 | 0.5 | 0.20 | 0.0148 | 72 | 9.6 | 21.6 | 3.3 | 2 | 0.122 | 0.5 | 0.20 | 0.0124 | 44 |
|  |  |  |  |  |  |  | 0.25 | 0.0157 | 87 |  |  |  |  |  |  | 0.25 | 0.0132 | 58 |
|  |  |  |  |  |  |  | 0.30 | 0.0159 | 93 |  |  |  |  |  |  | 0.30 | 0.0135 | 64 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0161 | 60 |  |  |  |  |  | 5.0 | 0.20 | 0.0160 | 59 |
|  |  |  |  |  |  |  | 1.00 | 0.0158 | 83 |  |  |  |  |  |  | 1.00 | 0.0136 | 57 |
|  |  |  |  |  |  |  | 2.00 | 0.0149 | 81 |  |  |  |  |  |  | 2.00 | 0.0143 | 74 |
| 3000 | 14.3 | 30.0 | 5.7 | 17 | 0.462 | 0.5 | 0.20 | 0.0144 | 68 | 9.4 | 22.9 | 3.6 | 7 | 0.323 | 0.5 | 0.20 | 0.0127 | 48 |
|  |  |  |  |  |  |  | 0.25 | 0.0148 | 77 |  |  |  |  |  |  | 0.25 | 0.0130 | 55 |
|  |  |  |  |  |  |  | 0.30 | 0.0154 | 87 |  |  |  |  |  |  | 0.30 | 0.0139 | 69 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0146 | 45 |  |  |  |  |  | 5.0 | 0.20 | 0.0142 | 41 |
|  |  |  |  |  |  |  | 1.00 | 0.0133 | 54 |  |  |  |  |  |  | 1.00 | 0.0139 | 61 |
|  |  |  |  |  |  |  | 2.00 | 0.0122 | 49 |  |  |  |  |  |  | 2.00 | 0.0142 | 74 |
| 5000 | 16.3 | 30.0 | 6.1 | 24 | 0.614 | 0.5 | 0.20 | 0.0140 | 63 | 10.3 | 22.5 | 4.1 | 11 | 0.492 | 0.5 | 0.20 | 0.0136 | 59 |
|  |  |  |  |  |  |  | 0.25 | 0.0149 | 77 |  |  |  |  |  |  | 0.25 | 0.0141 | 68 |
|  |  |  |  |  |  |  | 0.30 | 0.0156 | 89 |  |  |  |  |  |  | 0.30 | 0.0143 | 74 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0145 | 44 |  |  |  |  |  | 5.0 | 0.20 | 0.0143 | 42 |
|  |  |  |  |  |  |  | 1.00 | 0.0138 | 60 |  |  |  |  |  |  | 1.00 | 0.0138 | 60 |
|  |  |  |  |  |  |  | 2.00 | 0.0135 | 64 |  |  |  |  |  |  | 2.00 | 0.0137 | 67 |
| 7500 | 17.4 | 29.0 | 6.0 | 31 | 0.774 | 0.5 | 0.20 | 0.0114 | 33 | 10.2 | 22.8 | 3.9 | 17 | 0.665 | 0.5 | 0.20 | 0.0147 | 71 |
|  |  |  |  |  |  |  | 0.25 | 0.0126 | 50 |  |  |  |  |  |  | 0.25 | 0.0156 | 86 |
|  |  |  |  |  |  |  | 0.30 | 0.0145 | 76 |  |  |  |  |  |  | 0.30 | 0.0163 | 98 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0152 | 51 |  |  |  |  |  | 5.0 | 0.20 | 0.0147 | 46 |
|  |  |  |  |  |  |  | 1.00 | 0.0140 | 62 |  |  |  |  |  |  | 1.00 | 0.0142 | 64 |
|  |  |  |  |  |  |  | 2.00 | 0.0136 | 66 |  |  |  |  |  |  | 2.00 | 0.0141 | 72 |
| 10000 | 18.8 | 29.6 | 5.6 | 33 | 0.844 | 0.5 | 0.20 | 0.0113 | 31 | 12.0 | 24.3 | 4.8 | 21 | 0.741 | 0.5 | 0.20 | 0.0129 | 50 |
|  |  |  |  |  |  |  | 0.25 | 0.0135 | 61 |  |  |  |  |  |  | 0.25 | 0.0140 | 67 |
|  |  |  |  |  |  |  | 0.30 | 0.0155 | 89 |  |  |  |  |  |  | 0.30 | 0.0157 | 91 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0152 | 51 |  |  |  |  |  | 5.0 | 0.20 | 0.0143 | 42 |
|  |  |  |  |  |  |  | 1.00 | 0.0144 | 67 |  |  |  |  |  |  | 1.00 | 0.0134 | 55 |
|  |  |  |  |  |  |  | 2.00 | 0.0139 | 70 |  |  |  |  |  |  | 2.00 | 0.0135 | 65 |
| 12500 | 19.8 | 29.4 | 5.0 | 43 | 0.912 | 0.5 | 0.20 | 0.0117 | 36 | 12.0 | 25.3 | 4.7 | 28 | 0.848 | 0.5 | 0.20 | 0.0117 | 36 |
|  |  |  |  |  |  |  | 0.25 | 0.0136 | 63 |  |  |  |  |  |  | 0.25 | 0.0136 | 62 |
|  |  |  |  |  |  |  | 0.30 | 0.0148 | 80 |  |  |  |  |  |  | 0.30 | 0.0147 | 79 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0136 | 35 |  |  |  |  |  | 5.0 | 0.20 | 0.0143 | 42 |
|  |  |  |  |  |  |  | 1.00 | 0.0125 | 45 |  |  |  |  |  |  | 1.00 | 0.0136 | 57 |
|  |  |  |  |  |  |  | 2.00 | 0.0128 | 56 |  |  |  |  |  |  | 2.00 | 0.0139 | 70 |
| 15000 | 20.1 | 29.9 | 4.9 | 51 | 0.955 | 0.5 | 0.20 | 0.0106 | 24 | 13.2 | 23.8 | 4.8 | 30 | 0.873 | 0.5 | 0.20 | 0.0122 | 43 |
|  |  |  |  |  |  |  | 0.25 | 0.0131 | 57 |  |  |  |  |  |  | 0.25 | 0.0141 | 68 |
|  |  |  |  |  |  |  | 0.30 | 0.0145 | 76 |  |  |  |  |  |  | 0.30 | 0.0154 | 86 |
|  |  |  |  |  |  | 5.0 | 0.20 | 0.0153 | 52 |  |  |  |  |  | 5.0 | 0.20 | 0.0154 | 53 |
|  |  |  |  |  |  |  | 1.00 | 0.0144 | 67 |  |  |  |  |  |  | 1.00 | 0.0135 | 56 |
|  |  |  |  |  |  |  | 2.00 | 0.0159 | 94 |  |  |  |  |  |  | 2.00 | 0.0145 | 77 |
| 20000 | - | - | - | - | - | - | - | - | - | 14.0 | 24.6 | 4.4 | 37 | 0.938 | 0.5 | 0.20 | 0.0117 | 36 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.25 | 0.0132 | 58 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.30 | 0.0147 | 79 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.0 | 0.20 | 0.0152 | 51 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.00 | 0.0136 | 57 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.00 | 0.0135 | 65 |

Note: In the numerical simulation, the effect of non-uniform and high-density distribution of LF on flow resistance were quantified under different water depth and velocity according to real flow condition, which could extend to quantify the overall effect of roughness elements and flow conditions on flow resistance. The mean height H0 and the maximum height Hm are used to described the attachment thickness, mm; z is the surface vertical roughness, mm; Cb is the bed coverage, %; Lc is the roughness concentration; H is the water depth in the numerical simulation, m; U0 is the incoming velocity in the numerical simulation, m/s; n is the Manning’s n value; n is the change rate of Manning’s n value.